| AD Status report / Record of mandatory and non-mandatory tasks |  |  | File ref:G- |
| --- | --- | --- | --- |
|  |  |  | Sheet: 2 of: 3 |
| Registration: G | BGA Number: | Aircraft Type: Grob G102 | Serial Number: |
| Engine Type: N/A | Serial Number: N/A | Propeller Type: N/A | Serial number: N/A |

| Airworthiness Directive Number (AD) | Non mandatory task number (TN, SB) | Description | Method of compliance | Frequency (Hrs, Launches, calendar, one time) | Last complied with (Hrs, Launches, date) | Next due | Signature & date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LBA AD 83-91 | 306-22 | Inspection and exchange of the slide of pedal unit |  | One time |  | N/A |  |
| LBA AD 84-7 |  | Increase of service time | Superceded by LBA AD 93-41 | N/A | N/A | N/A |  |
| LBA AD 84-57 | 306-24 | Increase of service time | Superceded by LBA AD 93-41 | N/A | N/A | N/A |  |
| LBA AD 85-98 | 306-26 | Inspection / exchange of airbrake overcentre lever |  | One time |  | N/A |  |
| LBA AD 89-209 and 209/2 | 306-27/1 | Inspection of elevator hinges |  | One time |  | N/A |  |
| LBA AD 91-5 and 91-5/2 | 306-29 | Inspection and replacement of spar spigot assemblies |  | One time |  | N/A |  |
| LBA AD 92-356 | 306-31 | Inspection of airbrake stops | Inspection | Annual |  |  |  |
| LBA AD 93-001/3 |  | L'Hotellier connectors | Annual inspections | Annual |  |  |  |
| LBA AD 93-41 | 306-30 and 306-30/2 | Extension of service life | Inspections | 3000 hours |  |  |  |
| LBA AD 94-317/2 |  | Exchange of elevator lever |  | One time |  | N/A |  |
| LBA AD 2001- 317/4 | 306-36/3 | Increase of mass balance |  | One time |  | N/A |  |
| LBA AD 2004-168, & 168R1 | 306-38 | Inspection of rudder attachment | Annual inspection | Annual |  |  |  |

Enter details of the Airworthiness Directive or non mandatory task, brief description, brief method of compliance (inspection, replacement, service etc), frequency of the task (hours, launches, calendar time, one time), when the requirement was last complied with, when its next due (some tasks have more than one requirement) sign and date the entry. The signature does not constitute a certificate of release to service, this should be entered in the log book, Add new AD's when they are published even if compliance is due in the future.