‘Self=Declared Maintenance Programme’

BASED on EASA Part M Light (EU Reg. # 1321/2014)

Minimum Inspection Programme: -

WORKPACK Scheduled Maintenance [ 100 Hour / 200 Hour - Annual Inspection & 600 Hr] “Customised” for DIAMOND HK 36R aircraft

| Maintenance Organisation / Pilot-Owner / AME Name : Approval Reference AME No:/ BGA No. Site where maintenance being accomplished: | Workpack Ref:- Page 1 of 9 Note: Enter total pages of workpack issued |
| --- | --- |
| A/C Regn ; G- Type: DIAMOND HK 36R Serial No: Total Flying Hours: Engine Type- ROTAX 912 A Serial No: - Hours since new/overhaul Variable propeller type- MTV–1–A /170-8 Serial No: Hours since new/overhaul |  |
| Check being carried out :- 100 Flying Hours / 200 Flying Hours /600 Hr /OR Annual [ NOTE: - 50 Flying Hours Engine Oil Change, ONLY, if AVGAS (Leaded) used more than 30% of operation] Check Start Date: Completion Date: |  |

| Maintenance Manual References: Note: Maintenance Manuals must be to the latest revision | Issue / Revision No: | Date : |
| --- | --- | --- |
| Airframe: -DIAMOND HK 36R | Revision 10 | 17 May 1993 |
| Engine: - ROTAX 912 A | Edition # 1 | May 2007 |
| Variable propeller: - MTV-1-A/170-8 | Issue # 25 | 03 May 2004 |

CERTIFICATE OF RELEASE TO SERVICE. ‘Certifies that the work specified, except as otherwise specified, was carried out in accordance with Part M (M-L) and in respect to that work, the aircraft is considered ready for release to service.’ Sign....................... Name............................ Licence / Authorisation # .............................. Date........................

IAW MA.402 (f) ] UPON THE COMPLETION OF ALL MAINTENANCE, IT HAS BEEN VERIFIED THAT THE AIRCRAFT / COMPONENT IS CLEAR OF ALL TOOLS, EQUIPMENT AND ANY OTHER EXTRANEOUS PARTS OR MATERIAL AND THAT ALL ACCESS PANELS REMOVED, HAVE BEEN REFITTED. Authority # ............................ SIGNED ................................... Date:................................

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic | Qualifying Inspector |
| --- | --- | --- | --- | --- | --- |

Final Checks (Include with all checks except for the Pre-Flight Check and Check A)

GROUND RUN:

| 1 | Power plant, liquid, air and gas systems for leaks during and following ground run. | INSP | All Checks |  |  |
| --- | --- | --- | --- | --- | --- |
| 2 | Instruments, systems and services. Radio for electromagnetic interference. | OP/C | All checks |  |  |
| 3 | Following ground run, ensure all cowlings, access panels and doors are secure. | CHK | All checks |  |  |

CERTIFICATION:

| 4 | Workpack and Log Book entries have been completed and certified. Ensure items due in accordance with CAP 543 or the alternative document or system acceptable to the CAA, have been accomplished and certified. [Check compliance with AD’s & BGA TNS items] | CHK | All checks | N/A |  |
| --- | --- | --- | --- | --- | --- |

TYPE CERTIFICATE AND SCHEDULE REVIEW:

| 5 | Aeroplane complies with the type certificate data sheet. (TCDS A 065 Revision # 03 21 AUG 2015) | CHK | Annual | N/A |  |
| --- | --- | --- | --- | --- | --- |
| 6 | Mandatory placards are installed and legible. | CHK | Annual | N/A |  |

AIRFRAME INSPECTION ROUTINE

CABIN

| A01 | Check canopy for damage; check ‘locking device’ for serviceability | INSP | 100hr/200hr Annual & 600hr |  |  |
| --- | --- | --- | --- | --- | --- |
| A02 | Check canopy ‘Jettison device’ for serviceability | INSP | At 600hrs |  |  |
| A03 | Check seat belts & shoulder harnesses and their fastenings for damage | INSP | All checks |  |  |
| A04 | Check Elevator trim control, for damage interference, looseness of ‘notch plate’ and improper adjustment | INSP | All checks |  |  |
| A05 | Check Rudder pedals for corrosion and causticization, in particular, in the area of weld seams | INSP | All checks |  |  |
| A06 | Check Rudder pedals for looseness, interference and poor condition of ‘recuperator’ springs. Check pedal adjusting device for proper operation. (Lubricate if necessary) | INSP | All checks |  |  |
| A07 | Tailwheel Model: - Check brake pedals for interference, play, check mechanism for damage and wear. Check brake lines and brake cylinders for leakage. | INSP | All checks |  |  |
| A08 | Check brake fluid, (fill to maximum level 15 mm below top) | INSP | All checks |  |  |
| A09 | Check parking brake valve, operation and leaks. Bowden cable for correct adjustment. | INSP | All checks |  |  |
| A10 | Check control cables, in the area of ‘S-guides’ for broken strands and chafing. | INSP | All checks |  |  |

AIRFRAME INSPECTION ROUTINE (cont.)

CABIN

| A11 | Check control columns, for interference, defective stops and excessive play. Check zero (neutral) position | INSP | All checks |  |  |
| --- | --- | --- | --- | --- | --- |
| A12 | Measure the play, of the Aileron and Elevator control system, (with one stick blocked) Max allowable play: - 3 mm (1/8 in) Measured at the Trailing Edge of the surfaces. | INSP | All checks |  |  |
| A13 | Check the Air-Brake levers, for proper locking and unlocking, In the retracted position. Check air-brakes for asynchronous extension. | INSP | All checks |  |  |
| A14 | Check ALL Instruments, for incorrect or missing ‘Markings’ | INSP | All checks |  |  |
| A15 | Check all control knobs and handles for improper or missing inscriptions and designations. (Correct Colours) | INSP | All checks |  |  |
| A16 | Remove Instrument panel, Top Cover: - Check all electrical equipment, switches, instruments and circuit breakers, for secure mounting | INSP | All checks |  |  |
| A17 | Check the security of fuel valve mounting | INSP | All checks |  |  |
| A18 | Check the safe tying of Main Bolts for poor condition and serviceability | INSP | All checks |  |  |
| A19 | Check Main Bolts for interference. (Lubricate if necessary) | INSP | All checks |  |  |

STRUCTURE

| A20 | Check the skin of the wings, stabilizers and fuselage, for dents, cracks and holes | INSP | All checks |  |  |
| --- | --- | --- | --- | --- | --- |
| A21 | Check winglets, horizontal stabilizer tips and tail fins, for damage and looseness | INSP | All checks |  |  |
| A22 | Check aileron attachment and for excessive play in hinges. Check slot for defective or missing tape | INSP | All checks |  |  |
| A23 | Check Aileron bell crank and push rods (Through Inspection Window) for proper connection and safetying | INSP | All checks |  |  |
| A24 | Check Aileron webs for delamination and defective bonding | INSP | AT 600 hrs |  |  |
| A25 | Check Airbrakes and airbrake mechanism in wings and fuselage, for damage, improper connection and safetying | INSP | All checks |  |  |
| A26 | REMOVE WINGS | ................ | AT 200 hr Annual and 600 hr |  |  |
| A27 | Measure forces for locking, unlocking and extending the airbrakes. Lubricate Wing connecting bolts | INSP | AT 200 hr Annual and 600 hr |  |  |
| A28 | Check Main bulkhead inside, for damage and delamination | INSP | AT 200 hr Annual and 600 hr |  |  |
| A29 | Check wing spar stump for damage and delamination | INSP | AT 200 hr Annual and 600 hr |  |  |
| A30 | Check forward and rear web of wing spar, through holes in root rib and aileron inspection hole, for defective bonding, cracks and delamination. | INSP | AT 600 hrs |  |  |

AIRFRAME INSPECTION ROUTINE (cont.)

STRUCTURE

| A31 | Remove horizontal stabilizer, check fittings and locking ring, for security of attachment and corrosion. Check Locking function of Lock Ring. | INSP | AT 200 hr Annual and 600 hr |  |  |
| --- | --- | --- | --- | --- | --- |
| A32 | Check horizontal stabiliser and elevator for security of mounting. Check elevator hinges for secure attachment and safetying | INSP | All checks |  |  |
| A33 | Check both Rudder hinges for security of attachment and excessive play. Check rudder control cables, for damage, connection and looseness Check safetying of bolts on rudder lower mounting plate Check Lower edge of rudder for cracks and rubber marks | INSP | All checks |  |  |
| A34 | Check tail wheel steering mechanism and suspension. Check coil springs for play Check tail wheel fork and steering shaft, for deformation | INSP | All checks |  |  |
| A35 | Remove Rudder, relieve tail wheel, check lower rudder pedestal for deformation, cracks and corrosion | INSP | All checks |  |  |
| A36 | Remove tail wheel fork and check steering shaft for deformation. Remove Dowel Pins, extract hub of the tail wheel fork; check hub for deformation | INSP | AT 200 hr Annual and 600 hr |  |  |
| A37 | Tighten tail wheel rubber spring assembly | INSP | All checks |  |  |
| A38 | Lubricate rudder hinge bushes | LUB. | AT 200 hr Annual and 600 hr |  |  |
| A39 | Check vertical stabilizer stiffener and rearward ring frames, for condition, cracks and defective bonding | INSP | AT 600 hrs |  |  |
| A40 | Check rearward parts of elevator control for attachment, improper installation, excessive play, corrosion and improper safetying | INSP | All checks |  |  |
| A41 | Remove baggage compartment floor:- Check rudder control cables, rudder lever and aileron and airbrake control systems parts; for damage, corrosion, improper operation and safetying | INSP | All checks |  |  |
| A42 | Check forward ring frames, B-bulkhead and sickle shaped bulkhead, for delamination, cracks and defective bonding | INSP | AT 600 hrs |  |  |
| A43 | Remove seat shells, check area clean and NO loose objects, that may foul the controls | INSP | All checks |  |  |
| A44 | Check main bulkhead and forward and rearward transverse stiffener for delamination and defective bonding | INSP | All checks |  |  |
| A45 | Check firewall for delamination around engine mount attachment bolts | INSP | AT 600 hrs |  |  |
| A46 | Check fuel lines for leakage | INSP | All checks |  |  |
| A47 | Check electrical installation and ground straps for chafing; check wire attachments and connectors (by lightly pulling by hand) | INSP | All checks |  |  |
| A48 | Check Pitot tube and antennas for security | INSP | All checks |  |  |
| A49 | Check moisture drain holes and ventilation bores in wings, fuselage and control surfaces for blockage. | INSP | All checks |  |  |
| A50 | Check ALL cockpit placards are legible and comply to ‘Airplane Flight Manual’ (Also optional equipment) | INSP | All checks |  |  |
| A51 | Check aileron and airbrake control system parts in root ribs (wing & fuselage) for insecure mounting and wear. Lubricate as necessary | INSP | All checks |  |  |
| A52 | INSTALL WINGS | ................. | AT 200 hr Annual and 600 hr |  |  |

AIRFRAME INSPECTION ROUTINE (cont.)

STRUCTURE

| A53 | Check fuel quantity indicators for correct indication | INSP | AT 200 hr Annual and 600 hr |  |  |
| --- | --- | --- | --- | --- | --- |
| A54 | Check operation of stall warning system (if fitted) | INSP | All checks |  |  |

LANDING GEAR

| A55 | Clean landing gear, remove wheel fairings | INSP | All checks |  |  |
| --- | --- | --- | --- | --- | --- |
| A56 | Check main landing gear strut for cracks, deformation and damage | INSP | All checks |  |  |
| A57 | Check brake linings for wear. (Minimum thickness 3.5 mm (0.14 in.) | INSP | All checks |  |  |
| A58 | Check tyres for cuts, excessive wear and defective valve stems. Check creep marks. | INSP | All checks |  |  |
| A59 | Ensure correct tyre inflation pressure: - Main Gear. 2.1 bar (30 psi) Tailwheel. 3.1 bar (45psi) | INSP | All checks |  |  |
| A60 | Check rims for cracks: - Check bearings for rough running and play. Check brake discs for wear (Min. Thickness: 3.8 mm (0.15 in) | INSP | All checks |  |  |
| A61 | Remove main wheels, clean and lubricate bearings | INSP | AT 600 hrs |  |  |
| A62 | Check brake lines and brake cylinders for leaks, damage and corrosion. | INSP | All checks |  |  |
| A63 | Install wheels and wheel fairings, ensure secure attachment. | INSP | All checks |  |  |

LUBRICATION:

| A64 | Lubricate aeroplane in accordance with type design organisation recommendations. | CHK/LUB IAW Diamond M.M. 3.31 | As Required |  |  |
| --- | --- | --- | --- | --- | --- |

PROPELLER:

| P01 | Inspect propeller blades for ‘cracks in fibreglass sheath. See Maintenance Manual for ‘Limits’ and repair | INSP | 100 hr & ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |
| P02 | IAW EASA AD #2006-0345R1 and MT Service Bulletin # 8B (8 Mar. 2006) Inspect Blade Leading Edge Erosion Sheath for proper Bonding | INSP | 100 hr & ANNUAL |  |  |

‘EASA Part M Light’ MINIMUM INSPECTION PROGRAMME [ROTAX 912A]

ENGINE INSPECTION ROUTINE

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic | Qualifying Inspector |
| --- | --- | --- | --- | --- | --- |

| E01 | General visual inspection of engine; check cooling air duct and cooling fins of the cylinders for cracks. Take note of any signs of ‘fluid or gas’ leaks | INSP | 100 hr & ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |
| E02 | Visual inspection of ‘Temperature Sensor’ and ‘Oil Pressure Sensor’ for security and condition | INSP | 100 hr & ANNUAL |  |  |
| E03 | Inspect Coolant Hoses for damage, leakage, hardening from heat, porosity and loose connections’ Check routing is free from kinks and fretting. | INSP | 100 hr & ANNUAL |  |  |
| E03 | Check the base of ‘Water Pump’ for signs of leakage. | INSP | 100 hr & ANNUAL |  |  |
| E04 | Inspect the ‘Expansion Tank’ for damage. Check the Coolant Level and replenish as necessary. Inspect Radiator Cap. Inspect protective rubber at base of expansion tank, for correct fit. | INSP | 100 hr & ANNUAL |  |  |
| E05 | Inspect the ‘Overflow Bottle’ for damage. Check the line from the expansion tank to overflow bottle, for leaks, damage and clear flow. Check venting hole in overflow bottle cap. | INSP | 100 hr & ANNUAL |  |  |
| E06 | Inspect oil lines for damage, leakage, hardening from heat, porosity and security of attachment and connections. Check lines are free from kinks and restrictions. | INSP | 100 hr & ANNUAL |  |  |
| E07 | Inspect Fuel lines for damage, leakage, hardening from heat, porosity and security of attachment and connections. Check lines are free from kinks and restrictions. No cracks or scuffing marks | INSP | 100 hr & ANNUAL |  |  |
| E08 | Inspect Wiring and the connections for security, signs of wear and damage. | INSP | 100 hr & ANNUAL |  |  |
| E09 | Check the ‘Oil Filter’ for damage, tightness and abnormal wear. | INSP | 100 hr & ANNUAL |  |  |
| E10 | Check the ‘Magnetic Plug | INSP | 100 hr & ANNUAL |  |  |
| E11 | Carry out Cylinder Compression Check. (By Differential pressure method) | INSP | 200 hrs & Annual |  |  |

| Cylinder Pressure Drop. x/80 psi |  |  |  |  |
| --- | --- | --- | --- | --- |
| Cyl. # | 1 | 2 | 3 | 4 |
| Bar / Psi. | /80 | /80 | /80 | /80 |

POWERPLANT INSTALLATION:

| E12 | Inspect engine frame / suspension and fasteners for security, check for heat damage, deformation and cracks. | INSP | 100 hr & ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |
| E13 | Check Air Intake system for heat damage, deformation and cracks | INSP | 100 hr & ANNUAL |  |  |
| E14 | Inspect all external parts, for security of fit, inspect safety wiring and replace as necessary | INSP | 100 hr & ANNUAL |  |  |
| E15 | Check the Air Filter | INSP | 100 hr & ANNUAL |  |  |
| E16 | Clean engine on completion of maintenance | INSP | 100 hr & ANNUAL |  |  |

ENGINE INSPECTION ROUTINE

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic | Qualifying Inspector |
| --- | --- | --- | --- | --- | --- |

CARBURETORS

| E17 | Check Engine Idle Speed | INSP | 100 hr & ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |
| E18 | Check the Ventilation of the Float Chambers. Check vent lines free from kinks | INSP | 200 hrs & Annual |  |  |
| E19 | Check for ‘Free movement’ of throttle lever and starting carburettor. Check Bowden Cable allows full travel of the throttle lever ‘Stop to Stop’ | INSP | 100 hr & ANNUAL |  |  |
| E20 | Remove / Reinstall the two carburettors for inspection | INSP | 200 hrs & Annual |  |  |
| E21 | Check the pneumatic and mechanical synchronization of the carburettors | INSP | 100 hr & ANNUAL |  |  |
| E22 | Check the weight of ‘Floats’ | INSP | 200 hrs & Annual |  |  |
| E23 | Check the Carburettor sockets for abnormalities, wear, cracks | INSP | 200 hrs & Annual |  |  |

SPARK PLUGS

| E24 | SPARK PLUG connectors check a tight fit onto plug (Minimum pull-off force 30N. (7 lbs) | INSP | 200 hrs & Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| E25 | Remove all spark plugs, clean and gap electrodes Recommended replacement 200 hrs ( If Leaded fuel used 30% of operation) | INSP | 100 hr & ANNUAL |  |  |

COOLANT SYSTEM

| E26 | Flush the ‘Coolant System’ when replacing the coolant (When conventional coolants used) | INSP | At Coolant Replacement |  |  |
| --- | --- | --- | --- | --- | --- |

PROPELLER GEAR BOX

| E27 | Check the friction torque in free rotation, on gearboxes with overload clutch. | INSP | 100 hr & ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |

OIL CHANGE

| E28 | Drain Oil from tank Remove OLD oil filter and Install NEW filter [NOTE:- EVERY 50 hrs if LEADED FUEL USED MORE THAN 30% of OPERATION] | INSP | 100 hr & ANNUAL [See Note] |  |  |
| --- | --- | --- | --- | --- | --- |
| E29 | Check the Oil Tank and clean tank, if contaminated | INSP | 200 hrs & Annual |  |  |
| E30 | ‘CUT OPEN’ Oil Filter and ‘RECORD FINDINGS’ | INSP | 100 hr & ANNUAL |  |  |
| E31 | Replenish oil, ‘Verify’ Oil Level | INSP | 100 hr & ANNUAL |  |  |

Carry out engine ground run upon completion [Record operation, check for leaks]

DIAMOND HK 36R

Customised Maintenance Programme [ Review and Amend Annually ]

SEE CAP 543 (List of ‘Time Limited Tasks’ / Additional Inspections / Component Change Record)

Specific to Aircraft Registration / Serial Number.

LIST: - Of Any other Special Inspections / Recommendations required due specific Operating Environment and/or Storage conditions; as required by Owner / Operator.

| Document Reference # DIAMOND SB (etc) | Description of Task | Frequency of Inspection | COMPONENT ‘LIFE REMAINING’ [Flying Hours / Landings or Calendar Date.] | Certify Completion |
| --- | --- | --- | --- | --- |
| SB 25/1 | Carry out TBO replacement / inspection IAW SB | AT 3000 hrs |  |  |
| MSB 36-087/1 | Airframe ‘Life Extension’ [Replace ALL 3000 hr TBO components, as listed in Diamond S/B 25/1] | AT 6000 hrs |  |  |
| MSB 36-72 (AD 111) | Aileron Pushrods (Rod Ends) Inspection Repeat Annually until replacement | ANNUAL |  |  |
| SB 67 | Inspect Main Bolt ‘Safety Locl’ | EACH A/C RIG |  |  |
| SB 62 | Tailwheel assembly Inspection | ANNUAL |  |  |
| SB 39 | Tailwheel Rubber Discs | ANNUAL |  |  |
|  | Pilot Seat Harnesses. [Condition / Locking] | .Daily Inspection ‘LIFE’ On Condition |  |  |
|  | Compass Swing (Deviation Record) | Repeat 3 Yearly |  |  |
|  | Aircraft ‘Weight & Balance’ [Repeat every 8 Years] | Repeat 8 Yearly |  |  |
| ROTAX 912A SB 912-041R2 & 912-057 | Engine TBO. (ON CONDITION) Providing all AD’s & SB’s complied with and IAW CAA CAP 747 GR #24. | On Condition |  |  |
| EASA AD 2006-0316R1 (SB 912-051) | Inspection of ‘Magnetic Plug’ | 100 Hr &/or ANNUAL |  |  |
| EASA AD 2007-0025 (SB 912-029/R3) | Inspection of Crankcase for cracks | 100 Hr &/or ANNUAL |  |  |
|  | Flexible Hoses (Fuel & Oil) 5 Year Life. [Unless TEFLON] | 5Years |  |  |
| MTV VP Propeller | V.P. Propeller Overhaul (6Years / 72 months) |  |  |  |
| EASA AD 2006-034 | Repeat Inspection of L/E Metal Sheathing for Delamination | 100 Hr &/or ANNUAL |  |  |

|  | INITIAL / DUPLICATE INSPECTION of CONTROLS | Date / Sign Authorisation # | Date / Sign Authorisation # 2nd Inspection |  |
| --- | --- | --- | --- | --- |
| AILERON | Refit L/H: R/H: Correct Sense/Range of Movement |  |  |  |
| RUDDER | Refit: Correct Sense/Range of Movement |  |  |  |
| ELEVATOR | Refit: Correct Sense/Range of Movement |  |  |  |
| ENGINE CONTROLS | Rigging & Range of Movement |  |  |  |
|  | RECORD: - ALTIMETER & ASI CALIBRATION TEST RESULTS | ANNUAL |  |  |
|  | RECORD: - FLYING CONTROLS: RANGE of MOVEMENT’s | ANNUAL |  |  |

NOTE: - To Complete ‘Work Pack’

ATTACH,” SPARES ISSUED SHEETS” & ANY ADDITIONAL DEFECT / RECTIFICATION WORKSHHEETS.

This aircraft cannot be used for ‘Commercial Operations’. [Maintenance Programme to be reviewed Annually]

PILOT – OWNER MAINTENANCE. Shall be performed and certified: -in accordance with EASA Part M (M.L) M.A. 803; with limitations as listed, in Table Part A (AMC to Appendix VIII) [See also; BGA AMP 2–1 January 2016]

DECLARATION BY OWNER: - I hereby declare that this Maintenance Programme, is applicable to the aircraft referred to above and that I am fully responsible for its content and in particular any ‘Deviations’ introduced, as regards the Design Approved Holder recommendation. (TBO’s etc) SIGNED: - Pilot / Owner ...................................... ..Date:- .................