| AD Status report / Record of mandatory and non-mandatory tasks |  |  | File ref:G/CGBJ/2020/ |
| --- | --- | --- | --- |
|  |  |  | Sheet: of: |
| Registration: G-CGBJ | BGA Number: 3700 | Aircraft Type: ASTIR CS | Serial Number: 1107 |
| Engine Type: n/a | Serial Number: n/a | Propeller Type: n/a | Serial number: n/a |

| Airworthiness Directive Number (AD) | Non mandatory task number (TN, SB) | Description | Method of compliance | Frequency (Hrs, Launches, calendar, one time) | Last complied with (Hrs, Launches, date) | Next due | Signature & date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AD 81-267 | TN 306-18 | Service life extension |  | One time | 1981 | n/a |  |
| AD 85-98 | TN306-26 | Airbrake Bellcrank | Replacement | 3000 hrs | n/a | 3000 hrs |  |
| AD 89-209/2 | TN 306-27/1 | Elevator hinges | Inspection | One time | 10/5/91 | n/a |  |
| AD 91-5/2 | TN 306-29 | Spar Spigot | Inspection | One time | 26/01/91 | n/a |  |
| AD 93-41 | TN 306-30 | Service life extension |  | One time | 1993 | n/a |  |
| AD 93-41 | TN 306-30/2 | Airframe life 12000hrs | Inspection | One time | n/a | 12000 hrs |  |
| AD 93-41 | TN 306-30/2 | Intermediate life checks at 3000, 6000, then every 1000 to 9000, then every 500 to 12000 hrs | Inspection | At hours specified | n/a | 3000 hrs |  |
| AD 92-356 | TN 306-31 | Airbrake Stops | Inspection | Annually |  |  |  |
| AD 1993-001/3 | TN 306-32 | L'Hotellier connectors | Inspection | Annually |  |  |  |
| AD 94-317/2 | TN 306-33 | Elevator lever | Exchange | One time | 14/10/94 | n/a |  |
|  | TN 306-34 | Elevator moment limit |  | One time | 1994 | n/a |  |

Enter details of the Airworthiness Directive or non mandatory task, brief description, brief method of compliance (inspection, replacement, service etc), frequency of the task (hours, launches, calendar time, one time), when the requirement was last complied with, when its next due (some tasks have more than one requirement) sign and date the entry. The signature does not constitute a certificate of release to service, this should be entered in the log book, Add new AD's when they are published even if compliance is due in the future.