| AD Status report / Record of mandatory and non-mandatory tasks |  |  | File ref:G/CGBJ/2020/ |
| --- | --- | --- | --- |
|  |  |  | Sheet: of: |
| Registration: G-CGBJ | BGA Number: 3700 | Aircraft Type: ASTIR CS | Serial Number: 1107 |
| Engine Type: n/a | Serial Number: n/a | Propeller Type: n/a | Serial number: n/a |

| Airworthiness Directive Number (AD) | Non mandatory task number (TN, SB) | Description | Method of compliance | Frequency (Hrs, Launches, calendar, one time) | Last complied with (Hrs, Launches, date) | Next due | Signature & date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AD2006-0150R1 | MSB 306 - 38/2 | Tailplane Spherical Attachment Ball | Replacement | 10 yrs 1000 launch | 30/04/16 2829 | 30/04/26 3829 L |  |
| AD2006-0150R1 | MSB 306 - 38/2 | Tailplane Collar Bolts | Replacement | 10 yrs 1000 launch | 13/03/14 2743 L | 13/03/24 3743 L |  |
| AD2006-0150R1 | MSB 306 - 38/2 | Tailplane Attachment | Inspection | Annually |  |  |  |
|  | BGA011/12/2000 Iss 1 | Control tape | Inspection | Annually |  |  |  |
| AD 1998-018/3 |  | Nose release | Replacement | 2500 launch | 2504 L | 5004 L |  |
| AD 1998-018/3 |  | CofG release | Replacement | 2500 launch | 2093 L | 4593 L |  |
| AD 2010-0053 |  | Waterballast | Not fitted | n/a |  |  |  |
| AD 82 - 216 |  | Nicopress and Talurit cable sleves | Inspection | After cable change |  | After cable change |  |
|  | BGA056/08/2014 | Control Grips | Inspection | Annually |  |  |  |
|  | BGA Compendium | Check Flarm S/W up to date | nspection | Annually |  |  |  |

Enter details of the Airworthiness Directive or non mandatory task, brief description, brief method of compliance (inspection, replacement, service etc), frequency of the task (hours, launches, calendar time, one time), when the requirement was last complied with, when its next due (some tasks have more than one requirement) sign and date the entry. The signature does not constitute a certificate of release to service, this should be entered in the log book, Add new AD's when they are published even if compliance is due in the future.