| AD Status report / Record of mandatory and non-mandatory tasks |  |  | File ref:G/CGCT/2020/ |
| --- | --- | --- | --- |
|  |  |  | Sheet: 1 of: |
| Registration: G-CGCT | BGA Number: 3733 | Aircraft Type: Discus b | Serial Number: 360 |
| Engine Type: n/a | Serial Number: n/a | Propeller Type: n/a | Serial number: n/a |

| Airworthiness Directive Number (AD) | Non mandatory task number (TN, SB) | Description | Method of compliance | Frequency (Hrs, Launches, calendar, one time) | Last complied with (Hrs, Launches, date) | Next due | Signature & date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN360-1 | Fence Deflector | Modification | Optional | Not fitted | n/a |  |
|  | TN360-3 | Tail Wheel | Installation | Optional | Fitted | n/a |  |
|  | TN360-6 | Hinged Instrument Panel | Installation | Optional | Fitted | n/a |  |
|  | TN360-7 | Nose & CofG Hooks | Replacement | Optional | Fitted | n/a |  |
| AD 92-360/2 | TN360-9 | Elevator Drive Rod | Replacement | One Time | 26 Jan 1993 | n/a |  |
| AD 94-031 | TN360-11 | Service Life Increase to 12,000 | Inspection at set hrs | 6, 9, 10, 11, 12 K hrs | n/a | 6000 hrs |  |
|  | TN360-13 | Retrofit of Winglets | Modification | Optional | n/a | n/a |  |
|  | TN360-14 | Retrofit of Nose Hook | Modification | Optional | Fitted | n/a |  |
|  | TN360-22 | Emergency Bail Out System | Modification | Optional | n/a | n/a |  |
| MCA 10047913 | TN360-27 | Increase in Max Non-Lift Weight | Weight & Balance Calculation | As Required | 3 Feb 2019 | By 3 Feb 2027 |  |
| AD 98-018/3 |  | Nose Release | Replacement | 2500 launch | n/a | 2500 launch |  |
| AD 98-018/3 |  | CofG Release | Replacement | 2500 launch | n/a | 2500 launch |  |

Enter details of the Airworthiness Directive or non mandatory task, brief description, brief method of compliance (inspection, replacement, service etc), frequency of the task (hours, launches, calendar time, one time), when the requirement was last complied with, when its next due (some tasks have more than one requirement) sign and date the entry. The signature does not constitute a certificate of release to service, this should be entered in the log book, Add new AD's when they are published even if compliance is due in the future.