| AD Status report / Record of mandatory and non-mandatory tasks |  |  | File ref:G/DESU/2020/ |
| --- | --- | --- | --- |
|  |  |  | Sheet: of: |
| Registration: G-DESU | BGA Number: 2922 | Aircraft Type: ASK 21 | Serial Number: 21180 |
| Engine Type: n/a | Serial Number: n/a | Propeller Type: n/a | Serial number: n/a |

| Airworthiness Directive Number (AD) | Non mandatory task number (TN, SB) | Description | Method of compliance | Frequency (Hrs, Launches, calendar, one time) | Last complied with (Hrs, Launches, date) | Next due | Signature & date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN-2 | Inflateable Tail Wheel | Replacement | Fitted |  | n/a |  |
| AD 2013-0123 | TN-4b | Trim for Spin Instruction | Modification | Not Fitted | n/a | n/a |  |
|  | TN-5a | Toe Straps on Front Rudder Pedals | Modification | Fitted |  | n/a |  |
| AD1981-92 | TN-5b | Flight Manual | Amendment | Included in FM |  | n/a |  |
|  | TN-6 | Flight Manual | Amendment | Included in FM |  | n/a |  |
|  | TN-7 | Trim Ballast | Modification | Not Fitted | n/a | n/a |  |
|  | TN-8 | Flight Manual | Amendment | Included in FM |  | n/a |  |
| AD 1984-2 | TN-10 | Release Cable Fairlead | Modification | One Time | 26/02/2005 | n/a |  |
| AD 1984-32/2 | TN-13a | Flight Manual | Amendment (Rev 4 & 5) | Included in FM | 04/06/1984 | n/a |  |
|  | TN-14 | Maintenance Manual | Amendment | Included in MM |  | n/a |  |
| AD 1982-216 |  | Nicopress Sleeves | Inspection | After cable change | 26/02/2005 | n/a |  |
|  | TN-18 | Trim Indicator | Inspection | One Time | Not Fitted | n/a |  |

Enter details of the Airworthiness Directive or non mandatory task, brief description, brief method of compliance (inspection, replacement, service etc), frequency of the task (hours, launches, calendar time, one time), when the requirement was last complied with, when its next due (some tasks have more than one requirement) sign and date the entry. The signature does not constitute a certificate of release to service, this should be entered in the log book, Add new AD's when they are published even if compliance is due in the future.