Grob G 109 B : - ‘Self-Declared Maintenance Programme’

[ JW/TD/MIP/Grob 109 Iss 3 Rev. 0]

(Programme based on “Design Holder Data”)

WORKPACK [Scheduled Maintenance] [ 50 Hour / 100 Hour / Annual Inspection] for GROB G 109 B aircraft

| Maintenance Organisation / Pilot-Owner / AME Name: ……………………… Approval Reference AME No:/ BGA No. Site where maintenance being accomplished: | Work pack Ref: - Page 1 of 14 Note: Enter total pages of work pack issued [Spares Sheet & Additional Defect Tasks] |
| --- | --- |
| A/C Reg’n: - G- Type: GROB G109B Serial No: Total Flying Hours: Engine Type- GROB 2500 E1 /D1 Serial No: Hours since new/overhaul V/P propeller type- Hoffmann HO-V62R/L 160B Serial No: Hours since new/overhaul |  |
| Check being carried out 50 Hr / 100 Flying Hours / Annual Check Start Date: Completion Date: |  |

| Maintenance Manual References: - Note: Maintenance Manuals must be to the latest revision | Issue / Revision No: - | Date : - |
| --- | --- | --- |
| Airframe: GROB MM Dated 22 FEB 2002 | REVISION # 12 | 12 SEPT 2016 |
| Engine: GROB 2500 Dated JUL 1994 | ISSUE 2 | 01 JUL 1994 |
| Variable propeller: HOFFMAN HO-V62 [Manual # E 0107.72A] | NEW EDITION | AUG 2003 |

Permitted Variations to Maintenance Programme. (IAW. EASA / CAA) ‘FLYING HOURS’ : - (50 Hr / 100 Hr + 10%) (More than 5000 Hrs: + 500 Hrs) ‘CALENDAR TIME’ : - (1 Year or Less: + 10% or + 1 Month) (Up to 3 years: + 2 months) (More than 3 years: + 3 months) ‘LANDING CYCLES’ : - (500 Landings or Less. + 10% or + 25 Landings) (More than 500 Landings + 10% or + 500 Landings)

This aircraft cannot be used for ‘Commercial Operations’. [Maintenance Programme to be reviewed Annually] *[SEE PAGE 13 & 14 FOR LIST OF MAINTENANCE DEVIATIONS] *

‘DECLARATION BY OWNER’: - I hereby declare that this Maintenance Programme, is applicable to the aircraft referred to above and that I am fully responsible for its content and in particular any ‘Deviations’ introduced, as regards the Design Approved Holder recommendation. (TBO’s etc) SIGNED: - Pilot / Owner …................................................... Date: - …..............

PILOT – OWNER MAINTENANCE. Shall be performed and certified: -in accordance with EASA Part M (M.L) M.A. 803; with limitations as listed, in Table Part A (AMC to Appendix VIII) [See also; BGA AMP 2–1 Version 1 .2 1st October 2016]

Final Checks

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic | Qualifying Inspector |
| --- | --- | --- | --- | --- | --- |

Ground Run:

| 1 | Power plant, liquid, air and gas systems for leaks during and following ground run. | INSP | All Checks |  |  |
| --- | --- | --- | --- | --- | --- |
| 2 | Instruments, systems and services. Radio for electromagnetic interference. | OP/C | All checks |  |  |
| 3 | Following ground run, ensure all cowlings, access panels and doors are secure. | CHK | All checks |  |  |

Certification:

| 4 | Work pack and Log Book entries have been completed and certified. Ensure items due in accordance with BGA 280 / CAP 543 or the alternative document or system acceptable to the CAA, have been accomplished and certified. [Check compliance with AD’s & BGA TNS items] | CHK | All checks | N/A |  |
| --- | --- | --- | --- | --- | --- |

Type Certificate and Schedule Review:

| 5 | Aeroplane complies with the type certificate data sheet. (TCDS LBA 817 Revision # 11) | CHK | Annual | N/A |  |
| --- | --- | --- | --- | --- | --- |
| 6 | Mandatory placards are installed and legible. | CHK | Annual | N/A |  |

50 Flying Hour Inspection

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic | Qualifying Inspector |
| --- | --- | --- | --- | --- | --- |

Powerplant / Airframe

| A | Remove engine cowlings; inspect for fluid and gas leaks. Check Clamps / Hoses / Linkages / Engine Mounts. Drain and replenish engine oil. [Capacity MAX 3.5 Litres] | INSP & OP/C | 50 Hour |  |  |
| --- | --- | --- | --- | --- | --- |
| B | Replace oil filter element (Check filter for debris) | SERVICE | 50 Hour |  |  |
| C | Check Ignition Harness / Spark Plugs Check security of Starter / Alternator & Magneto. Check ‘V’ Belt Tension & Condition | INSP | 50 Hour |  |  |
| D | Carry out Airframe visual inspection: - [ Check Brake pad and Tyre wear, if aircraft used for intensive circuit training.] | INSP | 50 Hour |  |  |
| E | Carry out Visual Inspection of Hoffmann Propeller. (Stone Chip Damage / Delamination etc) Blade Root, ‘Silicon Seal’ condition. | INSP | 50 Hour |  |  |

CERTIFICATE OF RELEASE TO SERVICE. ‘Certifies that the work specified, except as otherwise specified, was carried out in accordance with Part M (M-L) [ BGA Approval. - UK MF. 0007] and in respect to that work, the aircraft is considered ready for release to service.’ Sign....................... Name............................ Licence / Authorisation # .............................. Date........................

IAW MA.402 (g) UPON THE COMPLETION OF ALL MAINTENANCE, IT HAS BEEN VERIFIED THAT THE AIRCRAFT / COMPONENT IS CLEAR OF ALL TOOLS, EQUIPMENT AND ANY OTHER EXTRANEOUS PARTS OR MATERIAL AND THAT ALL ACCESS PANELS REMOVED, HAVE BEEN REFITTED. Authority # ............................ SIGNED ................................... Date:...........................

100 Hrs & / or ANNUAL Inspection

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic | Qualifying Inspector |
| --- | --- | --- | --- | --- | --- |

Structural/Zonal:

| 7 | External structure of fuselage, main planes, empennage, cowlings, nacelles, control surfaces, flaps and other high lift devices. | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 8 | Normal and emergency doors and windows, door hinges, door hinge attachment points, required placards and operating instructions. | INSP | Annual |  |  |
| 9 | Doors, hatches and windows latching and locking. Replace Safety wire (0.4mm copper) after emergency jettison. | OP/C | Annual |  |  |

| 10 | Internal structure of fuselage, floors, bulkheads, main planes, nacelles, empennage. Control surfaces, flaps and other high lift devices, structural attachment joint assemblies, struts and their attachments. | INSP | 100 FH/ ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |
| 11 | Composite Construction: Vent holes, joints; bonded assemblies, protective treatments and finishes. | INSP | 100 FH/ ANNUAL |  |  |
| 12 | Corrosion protective treatments and drain holes. | INSP | 100 FH/ ANNUAL |  |  |

| 13 | Emergency exit by internal and external release methods. (Placards) | OP/C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 14 | Lightning strike bonding | CHK | Annual |  |  |

Landing Gear:

| 15 | Landing gear assemblies, shock absorber struts/units for leaks and correct extension, brake system, brake linings, drums/discs, wheels and tyres. | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 16 | Tyre pressures, hydraulic brake system fluid level. [Spec. DOT3 / DOT4] | SERVICE | Annual |  |  |

| 17 | Structural members, attachment fittings, pivot points, shock absorbing devices, torque links, shimmy dampers, main wheels, / tail wheels, bearings,, hoses and lines, and wheel fairing. [Note: Carry out with weight off the landing gear]. | INSP | 100 FH/ ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |
| 18 | Main and parking brake systems, | OP/C | Annual |  |  |

Flying Controls:

| 19 | Primary/secondary flight controls and trims systems for full and free movement in the correct sense. Position indicators agree with surface movement. | OP/C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |

| 20 | Hinges, brackets, push-pull rods, bell cranks, control horns, balance weights, pulleys, chains, tubes, guides, fairleads, rollers, tracks, rails, screw jacks/rams. [Note: The need for removal of flying control cables and control system components for detailed inspection must be assessed when accomplishing this task at the annual check]. | INSP | 100 FH/ ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |
| 21 | Check Controls Circuits, “Locking devices” in safety. | CHK | Annual |  |  |

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic | Qualifying Inspector |
| --- | --- | --- | --- | --- | --- |

Liquid, Air and Gas Systems:

| 22 | Hydraulic, pneumatic, vacuum and other fluid systems. | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 23 | Fluid levels in reservoirs | SERVICE | Annual |  |  |
| 24 | Pitot/static system vents, pitot head and drains clear. Pitot head correctly aligned. | OP/C | Annual |  |  |

| 25 | Pitot/static system sense and leak. | F/C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 26 | Flexible fuel and oil hoses (pressure test.) Note: In accordance with type design organisation pressure testing recommendations but in either case only until the ultimate service life, if stated, is achieved. | CHK | 100 FH/ ANNUAL |  |  |

Equipment and Environmental:

| 27 | Correct stowage of equipment, validity of date on emergency equipment. | CHK | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 28 | Seats, belts/harnesses, attachment, locking and release. | INSP | Annual |  |  |

| 29 | Cabin air system, heater | INSP & OP/C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |

Lubrication:

| 30 | Lubricate aeroplane in accordance with type design organisation recommendations. Lubricate Magneto Impulse with approx. 10 drops oil. | CHK/LUB | Annual |  |  |
| --- | --- | --- | --- | --- | --- |

Powerplant Installation:

| 31 | Engine and propeller controls for full and free movement – throttle, mixture, carburettor heat, and propeller pitch. | OP/C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 32 | Powerplant installation. [Check valve rocker clearance Cold (Inlet / Outlet 0.15mm / 0.006”] Remove & gap Spark Plugs (0.4mm / 0.016” Clean. Recommended Replace @ 150-200hrs] | INSP | 100 FH/ ANNUAL |  |  |

| 33 | Crankcase, accessory housings, cylinder assemblies, accessory drive belts, accessories, engine shock mounts, mount frames, bulkheads, firewalls and sealing, cooling baffles, cowlings, breathers and vents and items in engine bay for mutual interference. | INSP | 100 FH/ ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |

| 34 | Valve operating mechanism. Note: In accordance with type design organisation recommendations: | CHK | See Note |  |  |
| --- | --- | --- | --- | --- | --- |
| 35 | Cylinder compression and leakage. Record results below. Method: | CHK | Annual |  |  |

| Cylinder Pressure Drop. x/80 psi |  |  |  |  |
| --- | --- | --- | --- | --- |
| Cyl. # | 1 | 2 | 3 | 4 |
| Bar / Psi. | /80 | /80 | /80 | /80 |

Fuel Flow LPH

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic | Qualifying Inspector |
| --- | --- | --- | --- | --- | --- |

Air Induction:

| 36 | Air filter, intake and induction system and turbocharger impeller. [Air Filter-Clean Wire Mesh or Renew Filter Element (foam: mineral fibre)] | INSP | 100 FH/ ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |

| 37 | Carburettor heat, alternative air bypass doors and control system. | INSP & OP/C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 38 | Condition & security of hoses, any Drains clear. | INSP | Annual |  |  |

Exhaust:

| 39 | Exhaust manifold, mufflers. (Condition / leaks / security) | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |

| 40 | Exhaust, manifold Cabin heat exchanger. | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |

| 41 | Cabin heat exchanger pressure test. Note: In accordance with type design organisation recommendations.: | CHK | Annual |  |  |
| --- | --- | --- | --- | --- | --- |

Engine Lubrication:

| 42 | Engine oil change. Oil filter. Screen. Note: In accordance with type design organisation recommendations: [Capacity MAX 3.5 Litres] Next due: | SERVICE | 100 FH/ ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |

| 43 | Tanks, sumps, coolers, hoses, pipelines and vents. | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 44 | Engine controls in accordance with type design organisation recommendations. | LUB | Annual |  |  |

Fuel System:

| 45 | Filters for cleanliness and tank vents unobstructed. Drain samples from all drain points and check for presence of water, foreign matter and correct colour. | CHK | 100 FH/ ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |

| 46 | Tanks, filler cap, selector valve, pump, pipelines, hoses, carburettors, , throttle, mixture control, fuel selector control and filler point placard. | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |

Propeller:

| 47 | Blades, spinner and back plate Check for Blade L/E erosion. Check Blade Root “Silicon seal” condition | INSP | 100 FH/ ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |

| 48 | Pitch change mechanism for backlash. Check blade shake & Radial play angle up to 1.5deg. Check Thrust-plate wear/play Less than 0.2mm/ 0.008” Check ‘stop-nut’ torque 32.5lbs/ft (45Nm) | CHK | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 49 | Check (1mm) ‘Clearance’ between Torque Plate and Ball Bearing race. | LUB | Annual |  |  |

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic | Qualifying Inspector |
| --- | --- | --- | --- | --- | --- |

Ignition:

| 50 | Magnetos, harnesses, leads, switches, contact breakers, cooling systems and ventilators. | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 51 | Magneto internal timing and timing to engine. | CHK | Annual |  |  |
| 52 | Impulse Coupling / Magneto cam. Note: In accordance with type design organisation recommendations. | LUB | 100 FH/ ANNUAL |  |  |
| 53 | Spark plugs. Note: In accordance with type design organisation recommendations. | CHK | 100 FH/ ANNUAL |  |  |

Electrical System:

| 54 | Battery, stowage/compartment, vents and drains. Electrolyte level. | INSP & SERVICE | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 55 | Alternator/generator drive belt tension and condition. | INSP | Annual |  |  |

| 56 | Components, wiring, terminals and connectors. | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 57 | Correct type and rating of fuses and circuit breakers. Correct spare fuses carried. | CHK | Annual |  |  |
| 58 | Lamps and lighting. Correct spare lamps carried. | CHK | Annual |  |  |
| 59 | Brushes in starter, alternator and generator. Note: In accordance with type design organisation recommendations. | CHK | Annual |  |  |

| 60 | All ground operable electrical circuits. Exercise manually operated circuit breakers. | OP/C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 61 | Lead-acid battery capacity test. Note: In accordance with equipment manufacturer’s recommendations where capacity checks are recommended by the equipment manufacturer. | F/C | 12 months or See Note |  |  |

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic | Qualifying Inspector |
| --- | --- | --- | --- | --- | --- |

Avionics Radio:

| 69 | Aerials, insulators, controllers, instruments, displays, microphones, headsets, jack plugs and sockets. | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 70 | Cables and terminals, cooling systems and moisture trap areas. | INSP | Annual |  |  |
| 71 | VHF Communication. | OP/C | Annual |  |  |
| 72 | ATC Transponder using a field test set | F/C | 24 Months |  |  |
| 73 | Area and satellite navigation (GPS) | OP/C | Annual |  |  |
| 74 | Audio control panel, including emergency operation. | OP/C | Annual |  |  |
| 75 | VHF Communication system using a field test set, including frequency tolerance of transmitted frequencies. Note: In accordance with equipment manufacturer’s recommendations, only where frequency tolerance checks are recommended by the equipment manufacturer. Next due: | F/C | 36 months |  |  |
| 76 | Aerials and feeders – SWR (DME and ATC Transponder), insulation Next due: | F/C | 36 months |  |  |

Instrument Systems:

| 62 | Air speed indicator calibration (in situ is permissible). Note: Measured parameters must be recorded. | F/C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 63 | Altimeter calibration (in situ is permissible). Note: Measured parameters must be recorded. | F/C | Annual |  |  |
| 64 | Instruments and indicators for satisfactory condition, mounting, marking and operation. Note: This task is applicable to all instruments and indicators that could affect the airworthiness or operating safety of the aeroplane. | F/C | Annual |  |  |
| 65 | Readings consistent with ambient conditions. Stall warning device operation. | CHK | Annual |  |  |
| 66 | Compass ‘deviation’ or ‘steer by’ cards – valid until next check. | CHK | Annual |  |  |
| 67 | Instruments, displays, controllers, panels, mounts, pipes, hoses, electrical wiring, gyro filters, flux detectors, and instrument transmitters. | INSP | Annual |  |  |
| 68 | Compass ‘Check Swing’ | OP/C | 36 Months |  |  |

| ...................... | ............................................................... | ............................. | .................................... |
| --- | --- | --- | --- |
|  | INITIAL / DUPLICATE INSPECTION of CONTROLS | Date / Sign Authorisation # 1st Inspection | Date / Sign Authorisation # 2nd Inspection |
| AILERON | Refit L/H: R/H: Correct Sense/Range of Movement |  |  |
| RUDDER | Refit: Correct Sense/Range of Movement |  |  |
| AIR BRAKE | Refit: Correct Operation & Locking |  |  |

| ELEVATOR | Refit: Correct Sense/Range of Movement |  |  |
| --- | --- | --- | --- |
| ENGINE CONTROLS | Rigging & Range of Movement |  |  |
|  | RECORD: - ALTIMETER & ASI CALIBRATION TEST RESULTS | ANNUAL |  |
|  | RECORD: - FLYING CONTROLS: RANGE of MOVEMENT’s | ANNUAL |  |

GROB G 109B Customised Maintenance

Programme

[ Review and Amend Annually]

(List of ‘Time Limited Tasks’ / Additional Inspections / Component Change Record)

Specific to Aircraft Registration / Serial Number. [SEE also BGA Form 280 & CAP 543]

Mandatory Inspections in RED

LIST: - Of Any Other Special Inspections / Recommendations required due specific Operating Environment and/or Storage conditions; as required by Owner / Operator.

| Document Reference # LBA /EASA Ads GROB SB (etc) | Description of Task | Frequency of Inspection | COMPONENT ‘LIFE REMAINING’ [Flying Hours / Landings or Calendar Date.] | Certify Completion ‘SIGN’. |
| --- | --- | --- | --- | --- |
| TN 817 – 28/1 LBA 92 – 198 | AIRFRAME ‘Life Extension’ Inspection 3000 Hrs / 6000 / 7000 / 8000 - - Max 12000 Hrs | Wing Root Spar Fittings, Replace @3000 Hrs] |  |  |
| SB 817-29 LBA 90-315 | Wing Root Stud Plate Inspection (Loose Studs) | ANNUAL |  |  |
| SB 817-31 | Cushioned Tailwheel Assembly | One Time Only |  |  |
| SB 817-32 LBA 92-359 | Engine Exhaust System (Pressure Test) | Repeat 100 Hrs |  |  |
| SB 817-36 LBA 92-356 | Inspection Airbrake Stops | ANNUAL |  |  |
| TN 817 -39 LBA 94-004/2 | Main Undercarriage Leg Inspection [Pre-Mod Type. ’MPI’ NDT @ 1000 Landings] | Pre –Mod Type Only |  |  |
| SB 817-45 LBA 95-362 | Engine Mount Frame Inspection | ANNUAL |  |  |
| SB 817-53 | Inspection of R/H Control Stick | ANNUAL |  |  |
| SB 817-58 EASA 2012-0027 | Inspection of Vertical Stabilizer Nose Plate | Repeat 100 Hrs |  |  |
| SB 817-60 | Installation of Access Panel in Fin | One Time Only |  |  |
| SB 817-63 | Inspection of ‘Early Type’ Rudder Pedals | ANNUAL |  |  |
| SB 817-64 EASA 2012-0181 | Inspect Elevator Control Tube | Every 5 Years |  |  |
| MSB 817-70 EASA 2016-0228 | Tailwheel Mounting Bracket Inspection / Repair | Repeat 100 Hrs |  |  |
| --------------------- | --------------------------------- | ----------------- |  |  |
| Document Reference # LBA /EASA Ads GROB SB (etc) | Description of Task | Frequency of Inspection | COMPONENT ‘LIFE REMAINING’ [Flying Hours / Landings or Calendar Date.] | Certify Completion ‘SIGN’. |
| AD 1993-001/2 | L’Hotellier Connectors (Ball & Socket type) Inspection | ANNUAL |  |  |
| AD 1994-001/2 | L’Hotellier Connectors (Cam Locking type) Inspection | ANNUAL | N/A | N/A |
| SCHROTH Seat Buckles- SB 40 073 25-01 EASA AD 2017-0225 | SCHROTH Seat Restraint Buckle “Check Buckle Part No.” IAW SB. “NO AEROBATICS” Limitation | “One time Only” | N/A | N/A |
| B.G.A. TNS: - 12/97; 06/99; 12/00; 5/2010 | Control Surface ‘Tape’ and Seals Insp ‘Condition’ | 100 Hrs / Annual |  |  |
| B.G.A. TNS: - 5/2013 & AMP 4-8 | Pilot Seat Harnesses. [Condition / Locking] | Daily Inspection ‘LIFE’ On Condition |  |  |
| B.G.A. | Compass Swing (Deviation Record) | Repeat 3 Yearly |  |  |
| B.G.A. | Aircraft ‘Weight & Balance’ [Repeat every 8 Years] | Repeat 8 Yearly |  |  |
| B.G.A, | Instrument PVC Tubing Insp / Replace ‘Age Cracking’ | Repeat Annually |  |  |
| B.G.A, TNS: - 4-2014 | Control Grips (Check for Security) | Repeat Annually |  |  |

| Document Reference # LBA /EASA Ads GROB SB (etc) | ENGINE / PROPELLER | Frequency of Inspection | COMPONENT ‘LIFE REMAINING’ [Flying Hours / Landings or Calendar Date.] | Certify Completion ‘SIGN’. |
| --- | --- | --- | --- | --- |
| MSB 4601 -7/3 | Carburettor Float ’Needle Valve’ Replacement | 500 Hours Engine Operation |  |  |
| MSB 4601 -8 | Cylinder Head. ‘Valve Seat’ Inspection | 500 Hours Engine Operation |  |  |
| Airborne SL 58A | ‘Airborne’ Vacuum Pump | 500 Hours (or 6 Years) |  |  |
| ‘TEMPEST’ SL 003 | Check “Vane Wear” on Vacuum Pump | “On Condition” |  |  |
| Slick SB 1/89 | SLICK Magneto [DUAL MAGNETO: - ‘TBO’ 500 Hours] | 500 Hours |  |  |
| Slick SB 1/89 | SLICK Magneto [SINGLE MAGNETO: - ‘TBO’ 250 Hours] | 250 Hours |  |  |
| Bendix / Continental SB 643C | BENDIX Magneto 500 Hours or 4 Years since fitting | 500 Hours |  |  |
| HOFFMANN SB 61-10-03 E1 AK | VP Propeller ‘TBO ‘[ 72 Months or 1600 Hours] NOTE: - Maximum extension to 84 Months / 7 Years To coincide with next Annual Inspection. | 6 Years (72Months) |  |  |
| HOFFMANN SB E34, Rev B EASA 2017-0220 | VP Propellers Model HO-V62 “Check Blade Classification’ ‘A’ or ‘B’ Blades | ‘One Time Only’ Inspection |  |  |

| Document Reference # LBA /EASA Ads GROB SB (etc) | Description of Task | Frequency of Inspection | COMPONENT ‘LIFE REMAINING’ [Flying Hours / Landings or Calendar Date.] | Certify Completion ‘SIGN’. |
| --- | --- | --- | --- | --- |

‘EASA Part M Light’ MINIMUM INSPECTION PROGRAMME [GROB 109B]

Minimum Inspection Programme [ Review and Amend Annually]

MAINTENANCE ‘DEVIATION’S’ AGREED AND ACCEPTED BY PILOT / OWNER

| D # | SUBJECT | ALTERNATIVE MEANS of COMPLIANCE | Repeat Review Each 100 hrs and/or Annual | Sign/ Date |
| --- | --- | --- | --- | --- |
| D 01 | Hoffmann VP Propeller (TBO) | ‘Deviation’ with regard to Calendar time only. Due ‘Low annual utilisation’ and satisfactory hangar storage conditions; the repeat visual inspection (IAW M.I.P.) is deemed ‘safe operation’ beyond ‘Type Approval Holder’ recommendation. TBO recommendation. [Max 1600 Hrs &/or 72 months IAW SB 61-10-03 E 1 AK] [Note TCH Additional limitations “Aerobatic Hours”] |  |  |
| D 02 | Magnetic Compass | Repeat 3 yearly ‘Check swing’ can be dispensed with, (Day / VMC Flight) “Providing an alternative confirmation of Heading (I.E. GPS) is available and that ‘Magnetic Compass’ remains fully serviceable”. [EASA Part-NCO. AMC 1. NCO.IDE. A. 120 (a) & (1) NCO.IDE. A. 125 (a)] “Magnetic Compass or Equivalent” |  |  |
| D 03 | A/C Weighing | No ‘Changes to Aircraft’, Modification / Equipment; since Initial Weighing. (Any Weight change within +/ - 0.5% of Original Initial Weight.) The recommended 8-year repeat reweighing of the aircraft can be dispensed with, if the requirements of; [EASA Part-NCO Subpart C. GMI. NCO. POL. 105] are met |  |  |
| D 04 | Piston Engine (Overhaul Life) GROB 2500 E1/D1 | Recommended ‘TBO’ 1600 Hrs To enable ‘Extension’ of engine life, to “ON-CONDITION” all Mandatory AD’s must be complied with. [For Inspection Guidance, refer to CAA CAP 747 GR no.24] |  |  |
| D 05 | Magneto SLICK 4300 SB 1-89b | Recommended ‘TBO 250 Hrs (For SINGLE Magneto) 500 Hrs (For DUAL Magneto) Replace Magneto’s New at engine overhaul. Owner may extend ‘recommended TBO’ by Regular monitoring |  |  |
| D 06 | ‘Airborne’ Vacuum Pump E 211 CCS.L.58A | Recommended ‘TBO’ 500 Hrs or 6years from Manufacture Owner may extend ‘Life on-condition’ (Day VFR Flight only) |  |  |

‘EASA Part M Light’ MINIMUM INSPECTION PROGRAMME [GROB 109B]

Minimum Inspection Programme [ Review and Amend Annually]

MAINTENANCE ‘DEVIATION’S’ AGREED AND ACCEPTED BY PILOT / OWNER

| D # | SUBJECT | ALTERNATIVE MEANS of COMPLIANCE | Repeat Review Each 100 hrs and/or Annual | Sign/ Date |
| --- | --- | --- | --- | --- |
| D 07 | Fuel & Oil Flexible Hoses | Recommended 5 year ‘Replacement life’ Owner may extend life, by visual condition / flexibility / age cracks / pressure test. “TEFLON” hoses have extended life (Up to Engine Overhaul) |  |  |
| D 08 | Pilot “Seat -Harnesses’ | Recommended replacement life ’12 years’ BGA requirement (AMP 4-8) “on-condition” |  |  |

NOTE: - To Complete ‘Work Pack’ ATTACH,” SPARES ISSUED SHEETS” & ANY ADDITIONAL DEFECT / RECTIFICATION WORKSHHEETS