Minimum Inspection Programme: - Scheibe 28A

[Programme based on ‘Design Holder Data’]

WORK PACK : - Scheduled Maintenance [ 100 Hour / Annual Inspection] “Customised” for SCHEIBE SF28A aircraft

| Maintenance Organisation / Pilot-Owner / AME Name: Approval Reference AME No:/ Pilot Lic. / BGA No. Site where maintenance being accomplished: | Workpack Ref:- Page 1 of 9 Note: Enter total pages of work pack issued |
| --- | --- |
| A/C Reg’n.: - G- Type:- Scheibe 28A Serial No: Total Flying Hours: Engine Type- LIMBACH SL 1700 – EA1 Serial No: - Hours since new/overhaul Propeller type- HOFFMANN HO – V62/L 150A Serial No: Hours since new/overhaul |  |
| Check being carried out 100 Flying Hours / or Annual Check Start Date: Completion Date: |  |

| Maintenance Manual References: Note: Maintenance Manuals must be to the latest revision | Issue / Revision No: | Date : |
| --- | --- | --- |
| Airframe: - SCHEIBE FALKE | Issue 2 Oct 1978 |  |
| Engine: - LIMBACH - L 1700 | 7 / 92 Revision 2 (07 / 95) |  |
| Variable propeller: - HOFFMANN - E0107-72A | New Edition Aug 2003 |  |

PILOT – OWNER MAINTENANCE. Shall be performed and certified: -in accordance with EASA Part M (M.L) M.A. 803; with limitations as listed, in Table Part A (AMC to Appendix VIII) [See also; BGA AMP 2–1 January 2016] DECLARATION BY OWNER: - I hereby declare that this Maintenance Programme, is applicable to the aircraft referred to above and that I am fully responsible for its content and in particular any ‘Deviations’ introduced, as regards the Design Approved Holder recommendation. (TBO’s etc) SIGNED: - Pilot / Owner ............................................Date:- ................. This aircraft cannot be used for ‘Commercial Operations’. [Maintenance Programme to be reviewed Annually] *[SEE PAGE 9 FOR LIST OF MAINTENANCE DEVIATIONS] *

FINAL CHECKS: - (Include with all checks except for the Pre-Flight Check and Check A)

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic or PILOT/OWNER | Qualifying Inspector or PILOT/OWNER |
| --- | --- | --- | --- | --- | --- |

GROUND RUN:

| 1 | Power plant, liquid, air and gas systems for leaks during and following ground run. | INSP | All Checks |  |  |
| --- | --- | --- | --- | --- | --- |
| 2 | Instruments, systems and services. Radio for electromagnetic interference. | OP/C | All checks |  |  |
| 3 | Following ground run, ensure all cowlings, access panels and doors are secure. | CHK | All checks |  |  |

CERTIFICATION:

| 4 | Workpack and Log Book entries have been completed and certified. [Check compliance with AD’s & BGA TNS items] Ensure items due in accordance with CAP 543 or the alternative document or system acceptable to the CAA / EASA, have been accomplished and certified. | CHK | All checks | N/A |  |
| --- | --- | --- | --- | --- | --- |

TYPE CERTIFICATE AND SCHEDULE REVIEW:

| 5 | Aeroplane complies with the type certificate data sheet. (TCDS A 107 Rev. 1 (14 Sept 2006) | CHK | Annual | N/A |  |
| --- | --- | --- | --- | --- | --- |
| 6 | Review “Minimum Inspection Programme” for continued suitability against Aircraft ‘current usage’ /Storage conditions and any ‘Findings’ component failures. (UPDATE LIST OF ‘DEVIATIONS’ FROM RECOMMENDED MAINTENANCE INTERVALS; TBO’S ETC) | CHK | Annual | N/A |  |
| 7 | Mandatory placards remain installed and legible. | CHK | Annual | N/A |  |

Permitted Variations to Maintenance Programme. (IAW. EASA / CAA) ‘FLYING HOURS’ : - (50 Hr / 100 Hr + 10%) (More than 5000 Hrs: + 500 Hrs) ‘CALENDAR TIME’ : - (1 Year or Less: + 10% or + 1 Month) (Up to 3 years: + 2 months) (More than 3 years: + 3 months) ‘LANDING CYCLES’ : - (500 Landings or Less. + 10% or + 25 Landings) (More than 500 Landings + 10% or + 500 Landings)

CERTIFICATE OF RELEASE TO SERVICE. ‘Certifies that the work specified, except as otherwise specified, was carried out in accordance with Part M (M-L) and in respect to that work, the aircraft is considered ready for release to service.’ Sign....................... Name............................ Licence / Authorisation # .............................. Date........................

AIRFRAME INSPECTION ROUTINE

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic or PILOT/OWNER | Qualifying Inspector or PILOT/OWNER |
| --- | --- | --- | --- | --- | --- |

GENERAL

| All Tasks | Prior to inspection, ensure the aircraft is clean; Inspect for security, damage, integrity, drain/vent holes clear, signs of overheating, leaks, chafing and condition appropriate to the particular tasks. | INSP | All Checks |  |  |
| --- | --- | --- | --- | --- | --- |
| Lubrication / Servicing | Lubricate and Replenish fluids, in accordance with manufacturer’s requirements and ‘specifications’ | OP/C | All checks |  |  |
| Weighing | Review weighing record, to establish no changes against installed equipment. (Confirm in compliance with Reg’n Part – NCO) | CHK | Annual |  |  |

COCKPIT / FUSELAGE / WINGS

| A01 | Check Canopy door operation, Locking, integrity, emergency jettison. Condition of ‘glazing’. | INSP | 100hr/Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| A02 | Check Fuselage / & 4 Wing fittings for deformation, cracks and play. | INSP | 100hr/Annual |  |  |
| A03 | Check attachment bolts, for security & play (Max play 0.1mm) | INSP | 100hr/Annual |  |  |
| A04 | Check seat belts & shoulder harnesses as well as buckles and their fastenings for damage, satisfactory ‘locking’ and ‘unlocking’ | INSP | 100hr/Annual |  |  |
| A05 | Check Tailwheel steering; all controls on fuselage, wings, spoilers and Landing gear, for ease of movement, play and defects. | INSP | All checks |  |  |
| A06 | Check all connections of steering controls and spoilers for any defects, play and security. Check operation of Brake lever for ease of operation. | INSP | All checks |  |  |
| A07 | Check control cables (and controls), spoiler cables, wheel brake cable, for twists and wear (especially at Guides) | INSP | All checks |  |  |
| A08 | Check Rudder pedals for play and security | INSP | All checks |  |  |
| A09 | Check Rudder pedal adjustment, functional check | INSP | All checks |  |  |
| A10 | Check cable pulleys and guide connectors for play and wear | INSP | All checks |  |  |

AIRFRAME INSPECTION ROUTINE (Continued)

COCKPIT / FUSELAGE / WINGS

| A11 | Check Spoilers for symmetrical deployment (See Maintenance Manual for adjustment) | INSP | All checks |  |  |
| --- | --- | --- | --- | --- | --- |
| A12 | Check effectiveness of wheel brake. (Adjust if necessary) | INSP | All checks |  |  |
| A13 | Check all cockpit Instrument, for condition, function and correct marking. Including magnetic compass and radio. | INSP | All checks |  |  |
| A14 | Check mandatory placards are complete (ref. Operations Manual) | INSP | All checks |  |  |
| A15 | Check function of Fuel valve. | INSP | All checks |  |  |
| A16 | Check condition of Fabric covering and paint on wings, ailerons | INSP | All checks |  |  |
| A17 | Check fuselage for deformation, fabric covering and paint damage. (especially on Wheel cover and bottom of fuselage) | INSP | All checks |  |  |
| A18 | Check rudder and elevator for fabric , paint condition | INSP | All checks |  |  |
| A19 | Drain dynamic pressure hose | INSP | All checks |  |  |

| A20 | Check Pitot tube and hose assemblies for condition and security | INSP | All checks |  |  |
| --- | --- | --- | --- | --- | --- |
| A21 | Check fuselage frame and Landing gear, for damage. | INSP | All checks |  |  |
| A22 | Check wheels for play and condition | INSP | All checks |  |  |
| A23 | Tyre pressure (Fixed Single wheel 8.00x 4) 1.8 bar | INSP | All checks |  |  |
| A24 | Tyre pressure (tail wheel & stabilizers) 2.5 bar | INSP | All checks |  |  |
| A25 | Check ‘stabilizers’ and fittings for wear and condition. | INSP | All checks |  |  |
| A26 | Check all drainage holes for blockage (Especially in fuselage, landing gear, spoiler and lower rudder.) |  | All checks |  |  |
| A27 | Check aileron bearings for play (Axial & radial) | INSP | All checks |  |  |
| A28 | Check aileron gap to the wing ( also at full range of movement) | INSP | All checks |  |  |
| A29 | Check mounting of horizontal stabilizers | INSP | All checks |  |  |
| A30 | Check elevator bearings and trim hinges for wear and security. | INSP | All checks |  |  |
| A31 | Check Vertical fin for damage and security | INSP | All checks |  |  |
| A32 | Check Rudder bearings and drive for damage | INSP | All checks |  |  |
| A33 | Check Rudder and Elevator for sufficient gap, at full range of movement | INSP | All checks |  |  |

| A34 | Lubricate aeroplane in accordance with type design organisation recommendations. IAW SCHEIBE Check List (04/2012) | LUBRICATE | As Required |  |  |
| --- | --- | --- | --- | --- | --- |

‘EASA Part M Light’ MINIMUM INSPECTION PROGRAMME [Scheibe]

LIMBACH L 1700 ENGINE INSPECTION ROUTINE

| TASK No: | Task Description | Task Nature | Task Interval | Qualifying Mechanic or PILOT/OWNER | Qualifying Inspector or PILOT/OWNER |
| --- | --- | --- | --- | --- | --- |

| E01 | General visual inspection of engine; Take note of any signs of ‘fluid or gas’ leaks | INSP | 100 hr & ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |
| E02 | Remove Induction air filter; Clean or Replace | INSP | 100 hr & ANNUAL |  |  |
| E03 | Carburettor, check pushrods & joints for wear; Lubricate | INSP | 100 hr & ANNUAL |  |  |
| E04 | Remove fuel bowl; check Float and Needle valve; install with new seals | INSP | 100 hr |  |  |
| E05 | Dismantle and check carburettor diaphragm; replace as necessary | INSP | 100 hr & ANNUAL |  |  |
| E06 | Top up Carburettor Oil Level | INSP | 100 hr & ANNUAL |  |  |
| E07 | Check Fuel filter; remove and clean Check Fuel pump | INSP | 100 hr & ANNUAL |  |  |
| E08 | Check & Set Valve clearances (Inlet & Exhaust) | INSP | 100 hr & ANNUAL |  |  |
| E09 | Check Valve Shaft Projection Length (IAW Limbach AD 14) | INSP | 500Hrs |  |  |
| E10 | Check Alternator Drive Belt | INSP | 100 hr & ANNUAL |  |  |
| E11 | Check Spark Plugs. (Recommended replacement 150-200Hrs) | INSP | 100 hr & ANNUAL |  |  |
| E12 | Carry out Cylinder Compression Check. (By Differential pressure method) | INSP | 100 hrs & Annual |  |  |

| Cylinder Pressure Drop. x/80 psi |  |  |  |  |
| --- | --- | --- | --- | --- |
| Cyl. # | 1 | 2 | 3 | 4 |
| Bar / Psi. | /80 | /80 | /80 | /80 |

| E13 | Check Exhaust system | INSP | 100 hr & ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |
| E14 | Check all fuel & Oil Flexible hoses for condition. (Check in service Life limits Replace at 5Yrs.) | INSP | Annual |  |  |
| E15 | Check engine mount frame and attachment | INSP | 100 hr & ANNUAL |  |  |
| E16 | Inspect Magneto, check Timing and condition of Harness Check ‘Life’ as Slick recommended 250Hrs (See Slick SB 1-89B) | INSP | 100 hrs &Annual |  |  |
| E17 | Check all engine controls, full and free movement | INSP | 100 hr & ANNUAL |  |  |
| E18 | Check Starter motor security | INSP | Annual |  |  |
| E19 | Check alternator security | INSP | Annual |  |  |

‘EASA Part M Light’ MINIMUM INSPECTION PROGRAMME [Scheibe]

LIMBACH L 1700 ENGINE INSPECTION ROUTINE (Continued)

| E20 | Check condition and security of all electrical wiring | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| E21 | Drain engine Oil. Check oil Filter screen for any contamination (Metal Flakes) | INSP | 100 hr & ANNUAL |  |  |

| E22 | Replenish Engine Oil, with approved grade | INSP | Annual |  |  |
| --- | --- | --- | --- | --- | --- |

| E23 | Check engine Fuel Shut-Off Valve. Functional Check. [Note:-TM 653-70 has been complied with. 8Year Renewal N/A] | INSP | 100 hr & ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |

PROPELLER:

| P01 | Remove propeller spinner; clean propeller thoroughly | INSP | 100 hr & ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |
| P02 | Inspect Blade Leading Edge Erosion Sheath for proper Bonding [See info Hoffmann S.L. 61 – 12 – 04E 4 P. Date. May 21 2014] | INSP | 100 hr & ANNUAL |  |  |
| P03 | Check Hub Parts for cracks and correct fixing. [See S.B 61 – 11 – 03E 25A Date 05.Mar. 2013] Check Thrust ring wear | INSP | 100 hr & ANNUAL |  |  |
| P04 | Check Governor bearings, for free movement and wear. Check play between thrust ring and bearings | INSP | 100 hr & ANNUAL |  |  |
| P05 | Check Pitch Change for free movement. [See Also EASA AD 2005-370 & Hoffmann SB 61-11-03 E16) | INSP | 100 hr & ANNUAL |  |  |
| P06 | Check blade tracking | INSP | 100 hr & ANNUAL |  |  |
| P07 | Check compliance with Hoffmann SB’s (See Statement of ‘Deviation’ by Pilot/Owner, Page 9, “Deviation from Calendar time only”) [SB 61-10-03 E 1 AK 08 Dec 2008 Recommended “T.B.O,” 1000Hrs or 72Months] | INSP | 100 hr & ANNUAL |  |  |

(Carry out engine ground run upon completion [Record operation, check for leaks}

AVIONICS: - RADIO/ COMMS / NAV / TRANSPONDER

| R01 | Check VHF Communication | INSP | 100 hr & ANNUAL |  |  |
| --- | --- | --- | --- | --- | --- |
| R02 | Check ATC Transponder | INSP | 100 hr & ANNUAL |  |  |
| R03 | Check Aerials and cables | INSP | 100 hr & ANNUAL |  |  |

‘EASA Part M Light’ MINIMUM INSPECTION PROGRAMME [Scheibe]

Minimum Inspection Programme [ Review and Amend Annually ]

(List of ‘Time Limited Tasks’ / Additional Inspections / Component Change Record)

Specific to Aircraft Registration / Serial Number.)

LIST: - Of Any other Special Inspections / Deviation’s from Recommended ‘T.B.O. / Recommendations required due specific Operating Environment and/or Storage conditions; as determined by Owner / Operator.

| Document Reference #Scheibe SB &/or LIMBACH / Hoffmann | Description of Task | Frequency of Inspection | COMPONENT ‘LIFE REMAINING’ [Flying Hours / Landings or Calendar Date.] | Certify Completion |
| --- | --- | --- | --- | --- |
| Minor Change Approval A.C 10904 | Installation of Electrical Tachometer | ANNUAL |  |  |
| BGA | Inspection of ‘Wooden Structures’ / Fabric (Glue / Joint Failure) | ANNUAL |  |  |
| EASA AD # 2009-0132 | Elevator & Tab, Drive Arm of the Mechanical Elevator Trim Tab ‘Inspect / Repair | ANNUAL |  |  |
| BGA | Check Correct assembly of aircraft and functional check of controls. [Ensure fitting of ‘Wing Pins’ & Safety Lock Pin IAW TN 770-14] | EACH A/C RIG |  |  |
| LBA AD 1993-001/3 | L’Hotellier Ball and Socket Connectors with Lock Plates. Check wear & security of locking. | ANNUAL |  |  |
| LBA AD 1994-001/2 | L’Hotellier Ball and Socket Connectors (Type 45 ratchet). Check wear & Security of Locking. | ANNUAL |  |  |
| BGA | Pilot Seat Harnesses. [Condition / Locking] | Daily Inspection ‘LIFE’ On Condition |  |  |
| BGA | Compass Swing (Deviation Record) | Repeat 3 Yearly | Pilot / Owner ‘DEVIATION’ EASA Part-NCO |  |
| BGA | Aircraft ‘Weight & Balance’ [Repeat every 8 Years] See EASA Regulation Part-NCO | Repeat 8 Yearly | Pilot / Owner ‘DEVIATION’ EASA Part-NCO |  |
| SCHEIBE M.M. | Replacement of ‘Flexible fuel hoses’ in fuselage | 8 Years |  |  |
| CAA CAP 747 GR #24 | Engine TBO. (ON CONDITION) Providing all AD’s & SB’s complied with and IAW CAA CAP 747 GR #24. | On Condition |  |  |
| SLICK MAGNETO | Slick 4330 Magneto Inspect IAW SB’s Check Impulse Coupling. ‘Timing’ to engine. [Recommended Inspection / T.B.O. 250Hrs, if Single Mag. Or 500Hr Dual Mag Installation] | 100 Hr &/or ANNUAL |  |  |
| LIMBACH | Engine Bay Flexible Hoses (Fuel & Oil) 5 Year Life. [Unless TEFLON] | 5Years | See ‘Deviation’ D 04 |  |
| Hoffmann VP Propeller TBO | V.P. Propeller Overhaul (1000 Hrs / 6Years / 72 months) | Annual Inspection & review of condition and functional check | Pilot / Owner ‘DEVIATION’ from Design Approval Holder recommendation |  |

‘EASA Part M Light’ MINIMUM INSPECTION PROGRAMME [Scheibe]

Minimum Inspection Programme [ Review and Amend Annually ]

(continued)

|  | INITIAL / DUPLICATE INSPECTION of CONTROLS | Authorisation no. Sign / Date | 2nd Inspection Authorisation no. Sign / Date |
| --- | --- | --- | --- |
| AILERON | Refit: - L/H : R/H : Correct Sense/Range of Movement |  |  |
| RUDDER | Refit: - Correct Sense/Range of Movement |  |  |
| ELEVATOR | Refit: - Correct Sense/Range of Movement |  |  |
| ENGINE CONTROLS | Rigging / Adjustments & Range of Movement |  |  |
|  | RECORD: - ALTIMETER & ASI CALIBRATION TEST RESULTS | ANNUAL |  |
|  | RECORD: - FLYING CONTROLS: RANGE of MOVEMENT’s | ANNUAL |  |

IAW MA.402 (f) ] UPON THE COMPLETION OF ALL MAINTENANCE, IT HAS BEEN VERIFIED THAT THE AIRCRAFT / COMPONENT IS CLEAR OF ALL TOOLS, EQUIPMENT AND ANY OTHER EXTRANEOUS PARTS OR MATERIAL AND THAT ALL ACCESS PANELS REMOVED, HAVE BEEN REFITTED. Authority # ............................ SIGNED ................................... Date:................................

‘EASA Part M Light’ MINIMUM INSPECTION PROGRAMME [Scheibe]

Minimum Inspection Programme [ Review and Amend Annually ] (continued)

MAINTENANCE ‘DEVIATION’S’ AGREED AND ACCEPTED BY PILOT / OWNER

| # | SUBJECT | ALTERNATIVE MEANS of COMPLIANCE | Sign & Date |
| --- | --- | --- | --- |
| D 01 | Hoffmann VP Propeller (TBO) | ‘Deviation’ with regard to Calendar time only. Due ‘Low annual utilisation’ and satisfactory hangar storage conditions; the repeat visual inspection (IAW P01 to P07 M.I.P.) is deemed ‘safe operation’ beyond ‘Type Approval Holder’ recommendation. TBO recommendation. [Max 1000 Hrs &/or 72 months IAW SB 61-10-03 E 1 AK] |  |
| D 02 | Magnetic Compass | Repeat 3 yearly ‘Check swing’ can be dispensed with, (Day / VMC Flight) “Providing an alternative confirmation of Heading (I.E. GPS) is available and that ‘Magnetic Compass’ remains fully serviceable”. [EASA Part-NCO. AMC 1. NCO.IDE. A. 120 (a) & (1) NCO.IDE. A. 125 (a)] “Magnetic Compass or Equivalent” |  |
| D 03 | A/C Weighing | No ‘Changes to Aircraft’, Modification / Equipment; since Initial Weighing. (Any Weight change within +/ - 0.5% of Original Initial Weight.) The recommended 8-year repeat reweighing of the aircraft can be dispensed with, if the requirements of; [EASA Part-NCO Subpart C. GMI. NCO. POL. 105] are met |  |
| D 04 | Limbach Engine Flexible Hoses. (FUEL) | Extend ‘recommended’ 5-year Hose Life Visual Inspection Annually, remove Fire -proof sheath and check for surface age cracking / check hose flexibility; carry out a pressure leak test. Replace washers / seals at end fittings |  |

NOTE: - To Complete ‘Work Pack’

ATTACH,” SPARES ISSUED SHEETS” & ANY ADDITIONAL DEFECT / RECTIFICATION WORKSHHEETS.

Complete & Submit A.R.C.